SIMATS ENGINEERING (SAVEETHA SCHOOL OF ENGINEERING)

Department of Computer Science and Engineering

ASSESSMENT TOOL 2 – Short Answers

| Course Code: | ITA0325 | Course Title: | Mobile computing |
| --- | --- | --- | --- |
| CO Assessed: | CO1 | Assessment: | Assessment Tool 2 – Short Answers |
| Weightage: | 30 % of CIA | Total Marks: | 100 Marks |
| CO1 Statement: | Analyze the mobile network components, mobile base station, mobile infrastructures and protocols in cellular systems. |  |  |
| Student Name: | K Vamsi | Reg. No.: | 192372062 |
| Section / Batch: | CSE(AI)-2023 | Date: | 12/10/2026 |

Code of Conduct I K Vamsi / 192372062 (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment. I understand that any violation of academic integrity rules will result in disciplinary action. Signature of the Student: K Vamsi

Instructions - This test contains 50 questions, Total: 100 Marks. - Duration: 1 hour.

| Faculty In-charge | Course Coordinator | Head of Department |
| --- | --- | --- |
| Signature: _______________ | Signature: _______________ | Signature: _______________ |
| Date: ___________________ | Date: ___________________ | Date: ___________________ |

Questions

Q1. A field engineer downloads a 15 MB firmware file on a mobile link that sustains an average of 12 Mbps. How long will the download take?

Given:

- File size = 15 MB

- Download speed = 12 Mbps

Step 1: Convert megabytes into megabits.

We know that:

1 byte = 8 bits

Therefore,

File size = 15 × 8 = 120 Mb

Step 2: Apply the download-time formula.

Formula:

Download Time = File Size / Download Speed

Substituting the values:

Download Time = 120 Mb / 12 Mbps

Download Time = 10 seconds

Final Answer

Download time = 10 seconds

Q2. A background sync service on a smartphone draws an average current of 160 mA from a 4000 mAh battery. How many hours can the phone run this service before the battery is empty?

Given:

- Battery capacity = 4000 mAh

- Current consumption = 160 mA

Step 1: Apply the battery-life formula.

Formula:

Battery Life = Battery Capacity / Current Consumption

Substituting the values:

Battery Life = 4000 mAh / 160 mA

Battery Life = 25 hours

Final Answer: Battery life = 25 hours.

Q3. A handset sends a 1500-byte packet over a 2 Mbps uplink, and the propagation delay to the base station is 5 ms. What is the total one-way delay for the packet?

Given:

- Packet size = 1500 bytes

- Data rate = 2 Mbps

- Propagation delay = 5 ms

Step 1: Convert packet size into bits.

Packet Size = 1500 × 8

Packet Size = 12000 bits

Step 2: Calculate transmission delay.

Formula:

Transmission Delay = Packet Size / Data Rate

Transmission Delay = 12000 / 2000000 seconds

Transmission Delay = 0.006 seconds

Transmission Delay = 6 ms

Step 3: Calculate total delay.

Total Delay = Transmission Delay + Propagation Delay

Total Delay = 6 ms + 5 ms

Total Delay = 11 ms

Final Answer: Total delay = 11 ms.

Q4. A mobile app compresses a 4 MB photo by 75% before uploading it over a 400 kbps packet-data link. How long does the upload take?

Given:

- Original file size = 4 MB

- Compression = 75%

- Data rate = 400 kbps

Step 1: Calculate the remaining file size.

Remaining File Size = Original Size × (100 - Compression Percentage) / 100

Remaining File Size = 4 × (100 - 75) / 100

Remaining File Size = 1 MB

Step 2: Convert the file size into kilobits.

1 MB = 1000 KB

File Size = 1 × 1000 × 8

File Size = 8000 kilobits

Step 3: Calculate download time.

Download Time = File Size / Data Rate

Download Time = 8000 / 400

Download Time = 20 seconds

Final Answer: Download time = 20 seconds.

Q5. A planner locates co-channel cells by moving i = 2 cells along one axis and then j = 1 cell after a 60° turn. What cluster size N does this produce?

Given:

- i = 2

- j = 1

Step 1: Apply the cluster-size formula.

Formula:

N = i² + ij + j²

Step 2: Substitute the values.

N = 2² + (2 × 1) + 1²

N = 4 + 2 + 1

N = 7

Final Answer: Cluster size = 7 cells.

Q6. An operator adopts a 12-cell reuse cluster. What is the co-channel reuse ratio Q = D/R for this design?

Given:

- Cluster size, N = 12

Step 1: Apply the frequency-reuse formula.

Formula:

D / R = √(3N)

Where:

- D = Frequency reuse distance

- R = Cell radius

- N = Cluster size

Step 2: Substitute the values.

D / R = √(3 × 12)

D / R = √36

D / R = 6

Final Answer: Frequency reuse distance D = 6R.

Q7. Cells in a 7-cell cluster have a radius of 2 km. What is the minimum distance between the centres of two co-channel cells?

Given:

- Cluster size, N = 7

- Cell radius, R = 2 km

Step 1: Calculate the reuse ratio.

D / R = √(3N)

D / R = √(3 × 7)

D / R = √21

D / R = 4.583

Step 2: Calculate the reuse distance.

D = R × √(3N)

D = 2 × √21

D = 2 × 4.583

D = 9.17 km

Final Answer: Frequency reuse distance = 9.17 km.

Q8. A district of 1000 sq. km is to be covered with hexagonal cells of 1 km radius. Approximately how many cells are needed?

Given:

- District area = 1000 km²

- Cell radius, R = 1 km

Step 1: Calculate the area of one hexagonal cell.

Formula:

Cell Area = (3 × √3 / 2) × R²

Substituting the values:

Cell Area = (3 × √3 / 2) × 1²

Cell Area = 2.598 km²

Step 2: Calculate the number of cells.

Number of Cells = District Area / Cell Area

Number of Cells = 1000 / 2.598

Number of Cells = 384.91

Since a fraction of a cell cannot cover the whole district, round up.

Final Answer: Approximately 385 cells are required.

Q9. Each cell must support 50 full-duplex channels, each using 25 kHz in each direction, with a 7-cell reuse pattern. How much total spectrum does the operator need?

Given:

- Full-duplex channels per cell = 50

- Bandwidth per direction = 25 kHz

- Cluster size = 7 cells

Step 1: Calculate the bandwidth per full-duplex channel.

A full-duplex channel uses separate bandwidth for uplink and downlink.

Bandwidth per Duplex Channel = 25 + 25

Bandwidth per Duplex Channel = 50 kHz

Step 2: Calculate the bandwidth required per cell.

Bandwidth per Cell = Number of Channels × Bandwidth per Duplex Channel

Bandwidth per Cell = 50 × 50 kHz

Bandwidth per Cell = 2500 kHz

Bandwidth per Cell = 2.5 MHz

Step 3: Calculate the total bandwidth for the cluster.

Total Bandwidth = Bandwidth per Cell × Cluster Size

Total Bandwidth = 2.5 × 7

Total Bandwidth = 17.5 MHz

Final Answer: Total spectrum required = 17.5 MHz.

Note: This answer assumes that 25 kHz is allocated separately to each direction. If 25 kHz already represents the complete paired channel allocation, the result would be 8.75 MHz.

Q10. A town is served by 10 cells. Traffic growth forces every cell to be split into microcells of half the original radius. How many cells will cover the same area after splitting?

Given:

- Original number of cells = 10

- New cell radius = Half the original radius

Step 1: Understand the relationship between cell area and radius.

Cell Area is proportional to R².

If the radius becomes R / 2:

New Area / Original Area = (R / 2)² / R²

New Area / Original Area = 1 / 4

Each new cell covers one-fourth of the original area.

Step 2: Calculate the new number of cells.

New Number of Cells = Original Number of Cells × 4

New Number of Cells = 10 × 4

New Number of Cells = 40

Final Answer: 40 cells are required.

Q11. A cell with 60 channels is converted to three 120° sectors, with channels shared equally. How many channels does each sector get?

Given:

- Total channels per cell = 60

- Number of sectors = 3

Step 1: Apply the formula.

Channels per Sector = Total Channels / Number of Sectors

Channels per Sector = 60 / 3

Channels per Sector = 20

Final Answer: Each sector receives 20 channels.

Q12. A system has 1000 duplex channels and adopts a 4-cell cluster. How many channels does each cell receive?

Given:

- Total duplex channels = 1000

- Number of cells in the cluster = 4

Step 1: Calculate channels per cell.

Channels per Cell = Total Channels / Number of Cells

Channels per Cell = 1000 / 4

Channels per Cell = 250

Final Answer: Each cell receives 250 duplex channels.

Q13. A 7-cell reuse system operates where the path-loss exponent is 4. Considering only the six first-tier co-channel interferers, estimate the signal-to-interference ratio in dB.

Given:

- Cluster size, N = 7

- Path-loss exponent, n = 4

- Number of first-tier co-channel interferers = 6

Step 1: Calculate the reuse ratio.

D / R = √(3N)

D / R = √(3 × 7)

D / R = √21

Step 2: Calculate the signal-to-interference ratio.

Formula:

S/I = (D / R)ⁿ / 6

Substituting the values:

S/I = (√21)⁴ / 6

S/I = 441 / 6

S/I = 73.5

Step 3: Convert the ratio into decibels.

S/I in dB = 10 × log10(S/I)

S/I in dB = 10 × log10(73.5)

S/I in dB = 18.66 dB

Final Answer: S/I = 73.5, or approximately 18.66 dB.

Q14. A designer tests whether a 4-cell cluster can meet an 18 dB S/I target with a path-loss exponent of 4 and six first-tier interferers. What S/I does N = 4 actually give, and is the target met?

Given:

- Cluster size, N = 4

- Path-loss exponent, n = 4

- Number of interferers = 6

- Required S/I = 18 dB

Step 1: Calculate the reuse ratio.

D / R = √(3N)

D / R = √(3 × 4)

D / R = √12

Step 2: Calculate S/I.

S/I = (D / R)ⁿ / 6

S/I = (√12)⁴ / 6

S/I = 144 / 6

S/I = 24

Step 3: Convert to decibels.

S/I in dB = 10 × log10(24)

S/I in dB = 13.80 dB

Step 4: Compare with the requirement.

Required S/I = 18 dB

Calculated S/I = 13.80 dB

Shortfall = 18 - 13.80

Shortfall = 4.20 dB

Final Answer: The cluster size of 4 does not meet the 18 dB requirement. The S/I is approximately 13.80 dB.

Q15. A drive test records a serving signal of -70 dBm, co-channel interference of -88 dBm and a noise floor of -100 dBm. Compute the SINR in dB.

Given:

- Signal power = -70 dBm

- Interference power = -88 dBm

- Noise power = -100 dBm

Step 1: Convert the powers from dBm to milliwatts.

Formula:

Power in mW = 10^(Power in dBm / 10)

Signal Power = 10^(-70 / 10) mW

Signal Power = 1 × 10⁻⁷ mW

Interference Power = 10^(-88 / 10) mW

Interference Power ≈ 1.5849 × 10⁻⁹ mW

Noise Power = 10^(-100 / 10) mW

Noise Power = 1 × 10⁻¹⁰ mW

Step 2: Add interference and noise.

Total Interference and Noise = 1.5849 × 10⁻⁹ + 1 × 10⁻¹⁰

Total Interference and Noise = 1.6849 × 10⁻⁹ mW

Step 3: Calculate SINR.

SINR = Signal Power / (Interference Power + Noise Power)

SINR = (1 × 10⁻⁷) / (1.6849 × 10⁻⁹)

SINR ≈ 59.35

Step 4: Convert SINR to decibels.

SINR in dB = 10 × log10(59.35)

SINR ≈ 17.73 dB

Final Answer: SINR ≈ 17.73 dB.

Q16. A 1800 MHz carrier is used for a GSM rollout. What is its wavelength?

Given:

- Frequency, f = 1.8 GHz

- Speed of light, c = 3 × 10⁸ m/s

Step 1: Convert frequency to hertz.

f = 1.8 × 10⁹ Hz

Step 2: Apply the wavelength formula.

Wavelength = Speed of Light / Frequency

Wavelength = (3 × 10⁸) / (1.8 × 10⁹)

Wavelength = 0.1667 m

Step 3: Convert meters into centimeters.

Wavelength = 0.1667 × 100

Wavelength = 16.67 cm

Final Answer: Wavelength ≈ 0.167 m, or 16.67 cm.

Q17. At 900 MHz, at what distance does the free-space path loss reach 120 dB?

Given:

- Free-space path loss = 120 dB

- Frequency = 900 MHz

Step 1: Apply the free-space path-loss formula.

Formula:

FSPL = 32.44 + 20 × log10(f) + 20 × log10(d)

Where:

- f is frequency in MHz.

- d is distance in km.

Step 2: Substitute the values.

120 = 32.44 + 20 × log10(900) + 20 × log10(d)

120 = 32.44 + 59.08 + 20 × log10(d)

120 - 91.52 = 20 × log10(d)

28.48 = 20 × log10(d)

log10(d) = 28.48 / 20

log10(d) = 1.424

Step 3: Calculate the distance.

d = 10^1.424

d ≈ 26.55 km

Final Answer: Distance ≈ 26.55 km.

Q18. The received power at a 1 km reference distance is -50 dBm in an urban area with a path-loss exponent of 3.5. What power is expected at 4 km?

Given:

- Received power at 1 km = -50 dBm

- New distance = 4 km

- Path-loss exponent, n = 3.5

Step 1: Apply the log-distance path-loss formula.

Received Power at d = Received Power at d₀ - 10n × log10(d / d₀)

Here, d₀ = 1 km.

Step 2: Substitute the values.

Received Power = -50 - (10 × 3.5 × log10(4 / 1))

Received Power = -50 - (35 × 0.60206)

Received Power = -50 - 21.07

Received Power = -71.07 dBm

Final Answer: Received power at 4 km ≈ -71.07 dBm.

Q19. A 2100 MHz receiver can tolerate a maximum Doppler shift of 150 Hz. What is the highest vehicle speed, in km/h, the link can support?

Given:

- Carrier frequency = 2100 MHz

- Maximum Doppler shift = 150 Hz

- Speed of light = 3 × 10⁸ m/s

Step 1: Convert carrier frequency into hertz.

Carrier Frequency = 2100 × 10⁶ Hz

Step 2: Apply the Doppler formula.

Maximum Doppler Shift = (Mobile Speed × Carrier Frequency) / Speed of Light

Rearranging:

Mobile Speed = (Maximum Doppler Shift × Speed of Light) / Carrier Frequency

Step 3: Substitute the values.

Mobile Speed = (150 × 3 × 10⁸) / (2100 × 10⁶)

Mobile Speed = 21.43 m/s

Step 4: Convert meters per second into kilometers per hour.

Speed = 21.43 × 3.6

Speed = 77.14 km/h

Final Answer: Mobile speed ≈ 21.43 m/s, or 77.14 km/h.

Q20. A mobile channel has a maximum Doppler shift of 200 Hz. Using T = 0.423 / f(d), estimate the channel's coherence time.

Given:

- Maximum Doppler shift = 200 Hz

Step 1: Apply the coherence-time formula.

Coherence Time = 0.423 / Maximum Doppler Shift

Step 2: Substitute the value.

Coherence Time = 0.423 / 200

Coherence Time = 0.002115 seconds

Step 3: Convert seconds into milliseconds.

Coherence Time = 0.002115 × 1000

Coherence Time = 2.115 ms

Final Answer: Coherence time ≈ 2.115 ms.

Q21. A GSM operator has 25 MHz in each direction, uses 200 kHz carriers and leaves one carrier as a guard. Each carrier carries 8 time slots. How many traffic channels are available?

Given:

- Spectrum per direction = 25 MHz

- Carrier spacing = 200 kHz

- One carrier is reserved as a guard carrier

- Time slots per carrier = 8

Step 1: Convert spectrum into kilohertz.

25 MHz = 25000 kHz

Step 2: Calculate the number of carriers.

Number of Carriers = Total Spectrum / Carrier Spacing

Number of Carriers = 25000 / 200

Number of Carriers = 125

Step 3: Subtract the guard carrier.

Usable Carriers = 125 - 1

Usable Carriers = 124

Step 4: Calculate the number of time slots.

Total Time Slots = Usable Carriers × Slots per Carrier

Total Time Slots = 124 × 8

Total Time Slots = 992

Final Answer: 992 time slots.

Q22. A GSM burst period lasts 576.9 µs and the gross channel bit rate is 270.833 kbps. How many bits fit in one burst period?

Given:

- Gross bit rate = 270.833 kbps

- Burst period = 576.9 microseconds

Step 1: Convert the bit rate into bits per second.

Bit Rate = 270.833 × 1000

Bit Rate = 270833 bits/s

Step 2: Convert the burst period into seconds.

Burst Period = 576.9 × 10⁻⁶ seconds

Burst Period = 0.0005769 seconds

Step 3: Calculate the number of bit periods.

Number of Bit Periods = Bit Rate × Burst Period

Number of Bit Periods = 270833 × 0.0005769

Number of Bit Periods ≈ 156.25

Step 4: Interpret the result.

A GSM normal burst contains 148 transmitted bits, along with a guard period equivalent to approximately 8.25 bit periods.

Total Bit Periods = 148 + 8.25

Total Bit Periods = 156.25

Final Answer: Approximately 156.25 bit periods per burst, including the guard period.

Q23. An IS-95 CDMA link spreads a 9.6 kbps voice stream at a chip rate of 1.2288 Mcps. What is the processing gain in dB?

Given:

- Chip rate = 1.2288 Mcps

- Data rate = 9.6 kbps

Step 1: Convert the rates into consistent units.

Chip Rate = 1,228,800 chips/s

Data Rate = 9,600 bits/s

Step 2: Calculate the processing gain.

Processing Gain = Chip Rate / Data Rate

Processing Gain = 1,228,800 / 9,600

Processing Gain = 128

Step 3: Convert the processing gain into decibels.

Processing Gain in dB = 10 × log10(128)

Processing Gain in dB ≈ 21.07 dB

Final Answer: Processing gain = 128, or approximately 21.07 dB.

Q24. An FDMA system has 12.5 MHz of spectrum, 30 kHz channels and a 10 kHz guard band at each edge of the allocation. How many channels can it provide?

Given:

- Total spectrum = 12.5 MHz

- Channel bandwidth = 30 kHz

- Guard band at each outer edge = 10 kHz

Step 1: Convert the total spectrum into kilohertz.

Total Spectrum = 12.5 × 1000

Total Spectrum = 12500 kHz

Step 2: Calculate the usable spectrum.

Total Edge Guard Band = 10 + 10

Total Edge Guard Band = 20 kHz

Usable Spectrum = 12500 - 20

Usable Spectrum = 12480 kHz

Step 3: Calculate the number of channels.

Number of Channels = Usable Spectrum / Channel Bandwidth

Number of Channels = 12480 / 30

Number of Channels = 416

Final Answer: 416 channels.

Q25. An LTE carrier has 1200 usable subcarriers spaced 15 kHz apart, grouped 12 per resource block. How many resource blocks does the carrier offer?

Given:

- Total subcarriers = 1200

- Subcarriers per resource block = 12

- Bandwidth per resource block = 180 kHz

Step 1: Calculate the number of resource blocks.

Number of Resource Blocks = Total Subcarriers / Subcarriers per Resource Block

Number of Resource Blocks = 1200 / 12

Number of Resource Blocks = 100

Step 2: Calculate the occupied bandwidth.

Occupied Bandwidth = Number of Resource Blocks × Bandwidth per Resource Block

Occupied Bandwidth = 100 × 180 kHz

Occupied Bandwidth = 18000 kHz

Occupied Bandwidth = 18 MHz

Final Answer: 100 resource blocks, with an occupied bandwidth of 18 MHz.

Q26. Handsets contend on a slotted-ALOHA random access channel with an offered load of G = 0.5 attempts per slot. What throughput does the channel achieve?

Given:

- Offered traffic, G = 0.5 packets per slot

Step 1: Apply the slotted ALOHA throughput formula.

Throughput = G × e^(-G)

Where e is approximately 2.71828.

Step 2: Substitute the value.

Throughput = 0.5 × e^(-0.5)

Throughput = 0.5 × 0.6065

Throughput = 0.3033 packets per slot

Final Answer: Throughput ≈ 0.3033 packets per slot.

Q27. A location area has 25 cells, and the network must page 2000 incoming calls per hour in every cell of that area. How many paging messages are sent per hour?

Given:

- Number of cells = 25

- Incoming calls per hour per cell = 2000

- One paging message per cell for each incoming call

Step 1: Calculate the total paging messages.

Total Paging Messages = Number of Cells × Calls per Hour per Cell

Total Paging Messages = 25 × 2000

Total Paging Messages = 50000

Final Answer: 50000 paging messages per hour.

Q28. A BSC controls 40 BTSs. Each BTS has 3 sectors, each sector has 2 TRXs, and each TRX carries 8 time slots. How many time slots does the BSC manage?

Given:

- Number of BTSs = 40

- Sectors per BTS = 3

- TRXs per sector = 2

- Time slots per TRX = 8

Step 1: Calculate the total number of TRXs.

Total TRXs = BTSs × Sectors per BTS × TRXs per Sector

Total TRXs = 40 × 3 × 2

Total TRXs = 240

Step 2: Calculate the total number of time slots.

Total Time Slots = Total TRXs × Slots per TRX

Total Time Slots = 240 × 8

Total Time Slots = 1920

Final Answer: 1920 time slots.

Q29. One MSC can serve 500,000 subscribers. How many MSCs does a city with 2.4 million subscribers need?

Given:

- Total subscribers = 2.4 million

- Subscribers supported by one MSC = 500000

Step 1: Apply the formula.

Number of MSCs = Total Subscribers / Subscribers per MSC

Step 2: Substitute the values.

Number of MSCs = 2400000 / 500000

Number of MSCs = 4.8

Step 3: Round up to a whole number.

Number of MSCs Required = 5

Final Answer: 5 MSCs are required.

Q30. An Abis link runs on an E1 line of 2.048 Mbps, divided into 16 kbps sub-channels. How many sub-channels does the link carry?

Given:

- E1 data rate = 2.048 Mbps

- Required subchannel rate = 16 kbps

Step 1: Convert the E1 rate into kilobits per second.

E1 Data Rate = 2.048 × 1000

E1 Data Rate = 2048 kbps

Step 2: Calculate the number of subchannels.

Number of Subchannels = Total Data Rate / Rate per Subchannel

Number of Subchannels = 2048 / 16

Number of Subchannels = 128

Final Answer: The theoretical result is 128 subchannels of 16 kbps.

Q31. An HLR stores 1.5 KB of profile data per subscriber. How much storage is needed for 8 million subscribers?

Given:

- Data per subscriber = 1.5 KB

- Number of subscribers = 8 million

Step 1: Calculate the total data in kilobytes.

Total Data = Data per Subscriber × Number of Subscribers

Total Data = 1.5 × 8000000

Total Data = 12000000 KB

Step 2: Convert kilobytes into gigabytes using decimal units.

1 GB = 1000000 KB

Total Data = 12000000 / 1000000

Total Data = 12 GB

Final Answer: Total data transferred = 12 GB.

Q32. A VLR serves 60,000 visiting subscribers, and each performs 4 location updates per hour. How many location updates per second must the VLR process?

Given:

- Number of subscribers = 60000

- Updates per subscriber per hour = 4

Step 1: Calculate updates per hour.

Updates per Hour = Number of Subscribers × Updates per Subscriber

Updates per Hour = 60000 × 4

Updates per Hour = 240000

Step 2: Convert updates per hour into updates per second.

Updates per Second = Updates per Hour / 3600

Updates per Second = 240000 / 3600

Updates per Second = 66.67

Final Answer: Approximately 66.67 location updates per second.

Q33. A GSM full-rate speech codec outputs 13 kbps, and channel coding raises this to 22.8 kbps over the air. What percentage overhead does channel coding add?

Given:

- Original codec rate = 13 kbps

- Coded rate = 22.8 kbps

Step 1: Calculate the additional overhead rate.

Overhead Rate = Coded Rate - Original Rate

Overhead Rate = 22.8 - 13

Overhead Rate = 9.8 kbps

Step 2: Calculate overhead as a percentage of the original rate.

Overhead Percentage = (Overhead Rate / Original Rate) × 100

Overhead Percentage = (9.8 / 13) × 100

Overhead Percentage ≈ 75.38%

Final Answer: Codec overhead = 9.8 kbps, or approximately 75.38%.

Q34. A 1400-byte voice payload is carried with a 20-byte IP header, an 8-byte UDP header and a 12-byte RTP header. What is the protocol efficiency?

Given:

- Payload size = 1400 bytes

- IP header = 20 bytes

- UDP header = 8 bytes

- RTP header = 12 bytes

Step 1: Calculate the total header size.

Total Header = IP Header + UDP Header + RTP Header

Total Header = 20 + 8 + 12

Total Header = 40 bytes

Step 2: Calculate the total packet size.

Total Packet Size = Payload Size + Total Header

Total Packet Size = 1400 + 40

Total Packet Size = 1440 bytes

Step 3: Calculate packet efficiency.

Packet Efficiency = (Payload Size / Total Packet Size) × 100

Packet Efficiency = (1400 / 1440) × 100

Packet Efficiency ≈ 97.22%

Final Answer: Packet efficiency ≈ 97.22%.

Q35. An AMR codec produces a 244-bit speech frame every 20 ms, and each frame is sent with 40 bytes of IP/UDP/RTP headers. What bit rate does the stream use on the link?

Given:

- AMR frame size = 244 bits

- Frame duration = 20 ms

- Header size = 40 bytes

Step 1: Convert the header size into bits.

Header Size = 40 × 8

Header Size = 320 bits

Step 2: Calculate the total bits per frame.

Total Bits per Frame = AMR Frame Bits + Header Bits

Total Bits per Frame = 244 + 320

Total Bits per Frame = 564 bits

Step 3: Convert frame duration into seconds.

Frame Duration = 20 / 1000

Frame Duration = 0.02 seconds

Step 4: Calculate the total bit rate.

Total Bit Rate = Total Bits per Frame / Frame Duration

Total Bit Rate = 564 / 0.02

Total Bit Rate = 28200 bits/s

Total Bit Rate = 28.2 kbps

Final Answer: Total rate including the specified headers = 28.2 kbps.

Q36. In the packet core, a 1000-byte user packet is tunnelled with an 8-byte GTP-U header, an 8-byte UDP header and a 20-byte IP header. What percentage of the transmitted bytes is tunnelling overhead?

Given:

- Original packet size = 1000 bytes

- Additional UDP header = 8 bytes

- Additional outer IP header = 20 bytes

- Additional tunnel header = 8 bytes

Step 1: Calculate the total additional overhead.

Overhead = UDP Header + Outer IP Header + Tunnel Header

Overhead = 8 + 20 + 8

Overhead = 36 bytes

Step 2: Calculate the total packet size after tunneling.

Total Packet Size = Original Packet Size + Overhead

Total Packet Size = 1000 + 36

Total Packet Size = 1036 bytes

Step 3: Calculate overhead percentage.

Overhead Percentage = (Overhead / Total Packet Size) × 100

Overhead Percentage = (36 / 1036) × 100

Overhead Percentage ≈ 3.47%

Final Answer: Tunnel overhead = 36 bytes, or approximately 3.47% of the final packet size.

Q37. A GSM LAPDm signalling frame is 23 bytes long, of which 3 bytes are header. What is the frame's payload efficiency?

Given:

- Total frame size = 23 bytes

- Header size = 3 bytes

Step 1: Calculate the payload size.

Payload Size = Total Frame Size - Header Size

Payload Size = 23 - 3

Payload Size = 20 bytes

Step 2: Calculate frame efficiency.

Frame Efficiency = (Payload Size / Total Frame Size) × 100

Frame Efficiency = (20 / 23) × 100

Frame Efficiency ≈ 86.96%

Final Answer: LAPDm frame efficiency ≈ 86.96%.

Q38. A PLMN uses a 15-digit IMSI with a 3-digit MCC and a 2-digit MNC. How many digits are left for the MSIN, and how many subscribers can the operator number?

Given:

- Total IMSI length = 15 digits

- MCC length = 3 digits

- MNC length = 2 digits

Step 1: Calculate the number of MSIN digits.

MSIN Digits = Total IMSI Digits - MCC Digits - MNC Digits

MSIN Digits = 15 - 3 - 2

MSIN Digits = 10 digits

Step 2: Calculate the theoretical number of MSIN combinations.

Each digit has 10 possible values, from 0 to 9.

Number of Combinations = 10^10

Number of Combinations = 10000000000

Final Answer: Theoretical MSIN combinations = 10 billion.

Q39. Another operator in the same country is assigned a 3-digit MNC. How many unique subscriber identities can it issue?

Given:

- Total IMSI length = 15 digits

- MCC length = 3 digits

- MNC length = 3 digits

Step 1: Calculate the MSIN length.

MSIN Digits = Total IMSI Digits - MCC Digits - MNC Digits

MSIN Digits = 15 - 3 - 3

MSIN Digits = 9 digits

Step 2: Calculate the theoretical number of combinations.

Number of Combinations = 10^9

Number of Combinations = 1000000000

Final Answer: Theoretical MSIN combinations = 1 billion.

Q40. A core-network packet gateway handles 40 Gbps. In the busy hour, each user averages 2 Mbps when active and is active 10% of the time. How many users can the gateway support?

Given:

- Backhaul capacity = 40 Gbps

- Active-user data rate = 2 Mbps

- Fraction of users active at a time = 10%

Step 1: Convert the backhaul capacity into Mbps.

Backhaul Capacity = 40 × 1000

Backhaul Capacity = 40000 Mbps

Step 2: Calculate the average bandwidth required per subscriber.

Average Rate per Subscriber = Active-User Rate × Active Fraction

Average Rate per Subscriber = 2 × 0.10

Average Rate per Subscriber = 0.2 Mbps

Step 3: Calculate the number of supported subscribers.

Number of Subscribers = Backhaul Capacity / Average Rate per Subscriber

Number of Subscribers = 40000 / 0.2

Number of Subscribers = 200000

Final Answer: The theoretical estimate is 200000 subscribers.

Q41. On a data session, one-way delays are 10 ms on the radio link, 5 ms on backhaul, 3 ms in the core and 22 ms across the internet. What is the round-trip time?

Given:

- Core-network one-way delay = 10 ms

- Backhaul one-way delay = 5 ms

- Radio-network one-way delay = 3 ms

- Application-server one-way delay = 22 ms

Step 1: Calculate the total one-way delay.

One-Way Delay = Core Delay + Backhaul Delay + Radio Delay + Server Delay

One-Way Delay = 10 + 5 + 3 + 22

One-Way Delay = 40 ms

Step 2: Calculate the round-trip time.

Assuming the return path has the same delay:

Round-Trip Time = 2 × One-Way Delay

Round-Trip Time = 2 × 40

Round-Trip Time = 80 ms

Final Answer: One-way delay = 40 ms; round-trip time = 80 ms.

Q42. A network starts handoff when the received signal falls to -95 dBm, and the minimum usable signal for voice is -100 dBm. What is the handoff margin?

Given:

- Handoff trigger signal level = -95 dBm

- Minimum usable signal level = -100 dBm

Step 1: Calculate the handoff margin.

Handoff Margin = Handoff Trigger Level - Minimum Usable Level

Handoff Margin = (-95) - (-100)

Handoff Margin = 5 dB

Final Answer: Handoff margin = 5 dB.

Q43. A vehicle travels at 72 km/h along a highway lined with cells 2 km across. How many handoffs does it make in one hour?

Given:

- Mobile speed = 72 km/h

- Cell width = 2 km

Step 1: Calculate the distance travelled in one hour.

Distance Travelled = Speed × Time

Distance Travelled = 72 × 1

Distance Travelled = 72 km

Step 2: Calculate the number of cell widths crossed.

Cell Crossings per Hour = Distance Travelled / Cell Width

Cell Crossings per Hour = 72 / 2

Cell Crossings per Hour = 36

Final Answer: Approximately 36 cell crossings, and potentially 36 handoffs, per hour.

Q44. A handoff margin of 6 dB is set. The signal fades by 0.1 dB per metre near the cell edge, and the user moves at 15 m/s. How much time does the network have to complete the handoff?

Given:

- Fade margin = 6 dB

- Signal fade rate = 0.1 dB per meter

- Mobile speed = 15 m/s

Step 1: Calculate the distance available before the margin is exhausted.

Distance = Fade Margin / Fade Rate

Distance = 6 / 0.1

Distance = 60 m

Step 2: Calculate the time available.

Time = Distance / Speed

Time = 60 / 15

Time = 4 seconds

Final Answer: Available time = 4 seconds.

Q45. A serving cell is received at -85 dBm and a neighbour at -82 dBm, with a 4 dB handoff hysteresis. Will handoff be triggered? If not, how much stronger must the neighbour become?

Given:

- Serving-cell signal = -85 dBm

- Neighbor-cell signal = -82 dBm

- Hysteresis margin = 4 dB

Step 1: Calculate how much stronger the neighbor signal is.

Signal Difference = Neighbor Signal - Serving Signal

Signal Difference = (-82) - (-85)

Signal Difference = 3 dB

Step 2: Compare the difference with the hysteresis margin.

Required Difference = 4 dB

Actual Difference = 3 dB

Since 3 dB is less than 4 dB, the handoff condition is not satisfied.

Step 3: Calculate the additional improvement needed.

Additional Improvement = Required Difference - Actual Difference

Additional Improvement = 4 - 3

Additional Improvement = 1 dB

If the serving signal remains unchanged, the neighbor signal must reach -81 dBm to be 4 dB stronger.

Final Answer: Handoff should not occur yet. The neighbor signal must become 1 dB stronger to meet the hysteresis condition.

Q46. One million inbound roamers each make 2 location updates per day. Each update exchanges 5 signalling messages of 200 bytes. How much roaming signalling traffic is generated per day?

Given:

- Number of roaming subscribers = 1 million

- Updates per subscriber per day = 2

- Signaling messages per update = 5

- Size per message = 200 bytes

Step 1: Calculate the total updates per day.

Total Updates = Number of Subscribers × Updates per Subscriber

Total Updates = 1000000 × 2

Total Updates = 2000000 updates per day

Step 2: Calculate the data per update.

Data per Update = Messages per Update × Size per Message

Data per Update = 5 × 200

Data per Update = 1000 bytes

Step 3: Calculate total daily signaling data.

Total Data = Total Updates × Data per Update

Total Data = 2000000 × 1000

Total Data = 2000000000 bytes

Step 4: Convert bytes into gigabytes using decimal units.

Total Data = 2000000000 / 1000000000

Total Data = 2 GB

Final Answer: Daily signaling data = 2 GB.

Q47. A user crosses 12 cell boundaries per hour, and 20% of those crossings are also location-area boundaries. How many location updates does the user trigger per hour?

Given:

- Cell-boundary crossings per hour = 12

- Percentage of crossings at location-area boundaries = 20%

Step 1: Convert the percentage into a decimal.

Percentage = 20 / 100

Percentage = 0.20

Step 2: Calculate the expected location updates per hour.

Location Updates per Hour = Total Crossings × Boundary Percentage

Location Updates per Hour = 12 × 0.20

Location Updates per Hour = 2.4

Final Answer: Average location updates = 2.4 per hour.

Q48. A cell serves 2000 subscribers. Each makes 3 calls in the busy hour, and calls last 2 minutes on average. What traffic, in Erlangs, does the cell carry?

Given:

- Number of subscribers = 2000

- Calls per subscriber during the busy hour = 3

- Average call duration = 2 minutes

Step 1: Calculate the total number of calls per hour.

Total Calls per Hour = Number of Subscribers × Calls per Subscriber

Total Calls per Hour = 2000 × 3

Total Calls per Hour = 6000 calls per hour

Step 2: Convert average call duration into hours.

Average Call Duration = 2 / 60 hours

Average Call Duration = 1 / 30 hours

Step 3: Calculate offered traffic.

Offered Traffic = Calls per Hour × Average Call Duration in Hours

Offered Traffic = 6000 × (1 / 30)

Offered Traffic = 200 Erlangs

Final Answer: Offered traffic = 200 Erlangs.

Q49. Network statistics show 900 dropped calls out of 45,000 established calls in a day. What is the call drop rate?

Given:

- Dropped calls = 900

- Established calls = 45000

Step 1: Apply the call-drop-rate formula.

Call Drop Rate = (Dropped Calls / Established Calls) × 100

Step 2: Substitute the values.

Call Drop Rate = (900 / 45000) × 100

Call Drop Rate = 0.02 × 100

Call Drop Rate = 2%

Final Answer: Call drop rate = 2%.

Q50. A cell with 24 channels is offered 18 Erlangs at a 2% grade of service. How much traffic is actually carried, and what is the channel utilisation?

Given:

- Total channels = 24

- Offered traffic = 18 Erlangs

- Grade of Service (GoS) = 2% blocking probability

Step 1: Convert the blocking probability into a decimal.

Blocking Probability = 2 / 100

Blocking Probability = 0.02

Step 2: Calculate the carried traffic.

Assuming the stated 2% is the actual blocking probability:

Carried Traffic = Offered Traffic × (1 - Blocking Probability)

Carried Traffic = 18 × (1 - 0.02)

Carried Traffic = 18 × 0.98

Carried Traffic = 17.64 Erlangs

Step 3: Calculate channel utilization.

Maximum Channel Capacity = Number of Channels × 1 Erlang

Maximum Channel Capacity = 24 Erlangs

Channel Utilization = (Carried Traffic / Maximum Channel Capacity) × 100

Channel Utilization = (17.64 / 24) × 100

Channel Utilization = 73.5%

Final Answer: Carried traffic = 17.64 Erlangs, and channel utilization = 73.5%.